# Dot Hill Systems Storage Setup

# Prepare RAID Configuration Software

# Step.1: Install .NET Framework 3.5

  - Run the configuration tool, “ServerManager.exe”.

  - Select “Local Server” and click “Add Roles and Features”.

  - Click “Next”.

  - Click “Next”.

  - Select “Features” and check “.NET Framework 3.5”. Then click “Next”.

  - Click “Specify an alternate source path”.

  - Enter Source Path and click “OK”.

  - Click “Install”.

  - Click “Close”.

# Step.2: Install Universal RAID Utility Software to the database server

  - Put the EXPRESSBUILDER DVD-ROM medium into the DVD drive of the NEC Express5800/A2040b.

  - Run the setup program, “\002\win\uraidutl\Setup.exe”.

  - Click “Next”.

  - Click “Next”.

  - Click “Next”.

  - Click “Install”.

  - Click “Finish”.

# Configure RAID disks

# Step.1:  Alter the RAID System Management Mode

  - Run the configuration tool,
“C:\Program Files (x86)\Universal RAID Utility\gui\raidview.exe”.

  - Click “Tool” -> “Advanced Mode”.

  - Click “Yes”.

# Step.2:  Configure RAID disk for Data Array

# The figures in this section are examples. Number of RAID Controller, and capacity and number of the Physical Device and Logical Drive are not same as the benchmark configuration.

  - Select the first “LSI MegaRAID SAS 9286CV-8e”.

  - Click “Control” -> “Create Logical Drive” -> “Custom”.

  - Click “Add”.

  - Click “Next”.

  - Select “SAS-HDD(SSD)” and click “Add Disk Array”.

  - Check all Physical Devices and click “OK”.

  - Click “Next”.

  - Select “Write Through” as Cache Mode and click “OK”.

  - Click “Finish”.

  - Go back to the step 2-1 and repeat the same steps for the other “LSI MegaRAID SAS 9286CV-8e”s.