# Tier-B installation / configuration

# Tier-B hardware consists of one NEC Express5800/A2040b as the database server, two NEC Storage M300 and eleven Dot Hill 4120 as the Database Array and one NEC Express5800/R120d-2M as the System Console of the NEC Express5800/A2040b and the NEC Storage M300.

# The System Console (NEC Express5800/R120d-2M) has 1x Intel® Xeon® processor E5-2650, 2.00GHz, 20MB L3 cache, 24GB of Memory, 1x Internal SAS RAID Controller and 2x 147GB SAS drives, RAID1-protected, with Microsoft® Windows Server® 2012 Standard Edition. The machine is connected to the Management LAN ports of the Database Server via 1Gbps Ether switch, and connected to M300 controllers via 1Gbps Ether switch.

# Figure1.4: Rear view of the System Console (NEC Express5800/R120d-2M)

Connected to the Management LAN ports of the Database Server via 1Gbps Ether SW

Connected to M300 controllers　via 1Gbps Ether SW

# The NEC Express5800/A2040b has 4x Intel® Xeon® processor E7-4890 v2, 2.80GHz, 37.5MB L3 cache, 2048GB of Memory, 1x Internal SAS RAID Controller and 2x 450GB SAS drives, RAID1-protected, with Microsoft® Windows Server® 2012 Standard Edition. 1x 2port 8Gbps FC HBA, 11x External SAS RAID Controllers and 2x 2port 10Gbps Ether Controllers are installed to the PCI-Express slots of the NEC Express5800/A2040b. The FC HBA, External SAS RAID Controllers and Ether Controllers are connected to the Database Array as follows:

# MGB1 Management LAN			to the system console via 1Gbps Ether SW

# MGB2 Management LAN			to the system console via 1Gbps Ether SW

# PCI-Express #2: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #3: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #4: 2port 10Gbps Ether Controller	#0 to 10Gbps Ether port of client #2

# #1 to 10Gbps Ether port of client #1

# PCI-Express #5: 2port 10Gbps Ether Controller	#0 to 10Gbps Ether port of client #2

# #1 to 10Gbps Ether port of client #1

# PCI-Express #6: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #7: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #8: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #10: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #11: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #12: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #13: 2port 8Gbps FC HBA		#0 to M300 Controller

# #1 to M300 Controller

# PCI-Express #14: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #15: External SAS RAID Controller	to 4120 JBOD

# PCI-Express #16: External SAS RAID Controller	to 4120 JBOD

# Figure1.5: Rear view of the Server (NEC Express5800/A2040b)

# Figure1.6: Overview of the whole system connections