- Restore the tpch1g database and start 10 parallel streams of transactions. Wait for at least 100 iterations to complete.

- Clear the old IML and IEL logs in the iLO.

- List out the disks from OS and iLO that host the database data files.

- Abruptly power down the physical disk in Box 1, Bay 3 with S/N: S70XNE0T700221 hosting the data files. Please note that DSF with notation “/dev/nvmeXn1” is not persistent. The disks have to be mapped with S/N.

- Capture the disk loss from iLO IML

- Check if the OS is reporting IO errors on the concerned mount.

- Stop the client workload and backup the transaction log with no truncate.

- Forcefully drop the failed database. Drop database report error in removing the files from the missing disk, but successfully drops the database.

- Power the disk back up and capture the IML entry.

- Shutdown the database instance, restart the server and start the database instance back up.

- Restore the database tpch1g from its backup with no recovery clause.

- Restore the transaction log with recovery clause.

- Trigger the post run part of the Durability test from the StepMaster.