Durability Test Transcript : Transaction Log

This is to test resiliency of the database deployment when a RAID1 device (consisting of persistent memory devices) hosting database transaction log loses a mirror.

Major section in the test transcript:

Stage 1: Review the configuration

Stage 2: Invoke the required workload through StepMaster. Trigger the persistent memory failure when all streams complete at least 100 iterations:

Stage 3: Show availability of data with surviving mirror and recovery of the failed device:

Stage 1: Review the configuration

- Identify the path of transaction log:

- Identify the mount point and then the mount device:

Saleen380:~ # date;mount | grep log

Tue Nov 14 08:47:45 CST 2017

/dev/sdb on /var/opt/mssql/data/backup type xfs (rw,relatime,attr2,inode64,logbsize=256k,sunit=512,swidth=512,noquota)

/dev/md0 on /mnt/log type xfs (rw,noatime,attr2,inode64,noquota)

/dev/sdc on /mnt/data type xfs (rw,relatime,attr2,inode64,logbsize=256k,sunit=512,swidth=1536,noquota)

Saleen380:~ #

- “md0” is a software RAID1 device hosted by 2 persistent memory devices as indicated below:

Saleen380:~ # date;cat /proc/mdstat

Tue Nov 14 08:48:46 CST 2017

Personalities : [raid1]

md0 : active raid1 pmem2[0] pmem4[1]

106889216 blocks super 1.2 [2/2] [UU]

bitmap: 1/1 pages [4KB], 65536KB chunk

unused devices: <none>

Saleen380:~ #

- Query the status of the RAID device: CLEAN

Saleen380:~ # date;mdadm --detail /dev/md0

Tue Nov 14 08:48:54 CST 2017

/dev/md0:

Version : 1.2

Creation Time : Fri Oct 20 06:30:49 2017

Raid Level : raid1

Array Size : 106889216 (101.94 GiB 109.45 GB)

Used Dev Size : 106889216 (101.94 GiB 109.45 GB)

Raid Devices : 2

Total Devices : 2

Persistence : Superblock is persistent

Intent Bitmap : Internal

Update Time : Tue Nov 14 08:48:54 2017

State : clean

Active Devices : 2

Working Devices : 2

Failed Devices : 0

Spare Devices : 0

Consistency Policy : bitmap

Name : Saleen380:0  (local to host Saleen380)

UUID : bf7e61b6:5a437ef7:88d06324:0aaac331

Events : 100

Number   Major   Minor   RaidDevice State

0     259        0        0      active sync   /dev/pmem2

1     259        1        1      active sync   /dev/pmem4

Saleen380:~ #

- Identify the PMEM devices for error injection:

Saleen380:~/pmemeinj # date;ndctl list -n namespace4.0

Tue Nov 14 08:51:35 CST 2017

{

"dev":"namespace4.0",

"mode":"raw",

"size":109521666048,

"blockdev":"pmem4"

}

Saleen380:~/pmemeinj # date;./lsnvdimm -f -n

Tue Nov 14 08:51:39 CST 2017

-- NFIT TABLES --

NAME  PHYSID HANDLE SERIAL     FLAGS

nmem0 0x38   0xf0   0x04b22494

nmem1 0x39   0xf1   0x05317d82

nmem2 0x3a   0x10f0 0x0cfaf379

nmem3 0x3b   0x10f1 0x0deb2779

nmem4 0x37   0x2000 0x005868c0

-- NAMESPACES --

NAME              RESOURCE      SIZE         NUMANODE

namespace0.000000 0xa040000000  274877906944 0

namespace1.000000 0x59c0000000  302795194368 0

namespace2.000000 0x4040000000  109521666048 0

namespace3.000000 0x139c0000000 302795194368 1

namespace4.000000 0x12040000000 109521666048 1

Saleen380:~/pmemeinj #

- Display the script used for inducing the error

Saleen380:~/pmemeinj # cat date;cat pmemeinj_sqllog

cat: date: No such file or directory

#!/bin/bash

# IASL was previously installed on Rajesh's server.

cat /sys/firmware/acpi/tables/NFIT > /tmp/NFIT.dat

rm -f /tmp/NFIT.dsl

iasl -d /tmp/NFIT.dat 1>/dev/null 2>&1

grep "Address Range [BL]" /tmp/NFIT.dsl | sed 's/Address/PMEM Address/' | awk '{print $4 " " $5 " " $6 " " $7 $8 "\t" $9}'

echo "Injecting uncorrectable error in SQL /mnt/log pmem4 @ 0x12040000040..."

# Thierry will enable carefully these operations, one by one on Rajesh's server.

# For SQL LOG, inject a controller error to the physical nvdimm device mapping /dev/pmem4

# Refer to the lsnvdimm outputs for the mapping.

./dsmError -i -t 4 -p /dev/nmem3

./newndctl h n 20 /dev/nmem3

mount -t debugfs none /sys/kernel/debug

modprobe -v einj param_extension=1

cd /sys/kernel/debug/apei/einj

# cat available_error_type

echo 0x00000010 > error_type

echo 0 > notrigger

echo 0x12040000040 > param1

echo 0xfffffffffffff000 > param2

echo 1 > error_inject

dd if=/dev/pmem4 of=/dev/null iflag=direct

Saleen380:~/pmemeinj #

Stage2: Invoke the required workload through StepMaster. Trigger the persistent memory failure when all streams complete at least 100 iterations:

- Execute HPE script to induce uncorrectable errors and ensure that failure persists across reboots.

Saleen380:~/pmemeinj # date; ./pmemeinj_sqllog

Tue Nov 14 10:12:07 CST 2017

PMEM Address Range Base:        000000A040000000

PMEM Address Range Length:      0000004000000000

PMEM Address Range Base:        00000059C0000000

PMEM Address Range Length:      0000004680000000

PMEM Address Range Base:        0000004040000000

PMEM Address Range Length:      0000001980000000

PMEM Address Range Base:        00000139C0000000

PMEM Address Range Length:      0000004680000000

PMEM Address Range Base:        0000012040000000

PMEM Address Range Length:      0000001980000000

Injecting uncorrectable error in SQL /mnt/log pmem4 @ 0x12040000040...

dsmError - 0.76 -  -

version.c Build: Oct 18 2017 01:19:48

./dsmError: i:19, typ:4, opt:1, path=</dev/nmem3>

INJ ERR: Status: 0x0

newndctl - 0.76 -  -

version.c Build: Oct 18 2017 01:19:48

INJ ERR STATUS: Status: 0x0

Hex dump of data:

00000000

Error Injection Status Info = 0x2

0 No Error Injection Activity

1 Injected

0 Pending

mount: none is already mounted or /sys/kernel/debug busy

insmod /lib/modules/4.4.73-5-default/kernel/drivers/acpi/apei/einj.ko param_extension=1

---------Induced uncorrectable error causes a system panic followed by a reboot.

- This reboot triggers a backup of remaining functional persistent memory devices

- IM logs register the uncorrectable errors:

- Startup of the server proceeds by restoring Persistent memory devices:

- A warning on the uncorrectable error is issued

Stage3: Show availability of data with surviving mirror and recovery of the failed device:

- Check the software RAID device exists and subsequently query its health:

Saleen380:~ # date;ls /dev/md0

Tue Nov 14 10:22:13 CST 2017

/dev/md0

Saleen380:~ # date;cat /proc/mdstat

Tue Nov 14 10:22:19 CST 2017

Personalities : [raid1]

md0 : active (auto-read-only) raid1 pmem2[0]

106889216 blocks super 1.2 [2/1] [U_]

bitmap: 0/1 pages [0KB], 65536KB chunk

unused devices: <none>

Saleen380:~ # date;mdadm --detail /dev/md0

Tue Nov 14 10:22:26 CST 2017

/dev/md0:

Version : 1.2

Creation Time : Fri Oct 20 06:30:49 2017

Raid Level : raid1

Array Size : 106889216 (101.94 GiB 109.45 GB)

Used Dev Size : 106889216 (101.94 GiB 109.45 GB)

Raid Devices : 2

Total Devices : 1

Persistence : Superblock is persistent

Intent Bitmap : Internal

Update Time : Tue Nov 14 10:12:04 2017

State : clean, degraded

Active Devices : 1

Working Devices : 1

Failed Devices : 0

Spare Devices : 0

Consistency Policy : bitmap

Name : Saleen380:0  (local to host Saleen380)

UUID : bf7e61b6:5a437ef7:88d06324:0aaac331

Events : 100

Number   Major   Minor   RaidDevice State

0     259        4        0      active sync   /dev/pmem2

-       0        0        1      removed

- Check for data availability and reboot server for reviving the failed device:

Saleen380:~ # date;mount -t xfs -o rw,dax,noatime /dev/md0 /mnt/log

Tue Nov 14 10:22:38 CST 2017

Saleen380:~ # date;ls /mnt/log

Tue Nov 14 10:22:44 CST 2017

root_1G_.mdf  temp_file_log.log  tpch1g_log.mdf

Saleen380:~ # date;dd if=/dev/zero of=/dev/pmem4 bs=1024 count=1024

Tue Nov 14 10:22:59 CST 2017

dd: error writing '/dev/pmem4': Operation not permitted

1+0 records in

0+0 records out

0 bytes copied, 8.1935e-05 s, 0.0 kB/s

Saleen380:~ # date; shutdown -r now

Tue Nov 14 10:23:09 CST 2017

Connection to 10.116.38.131 closed by remote host.

Connection to 10.116.38.131 closed.

[tadakama@perf-lcoe11 ~]$

- Sanitize the failed device and add it back to the configuration:

Saleen380:~ # date; cat /proc/mdstat

Tue Nov 14 10:26:40 CST 2017

Personalities : [raid1]

md0 : active raid1 pmem2[0]

106889216 blocks super 1.2 [2/1] [U_]

bitmap: 1/1 pages [4KB], 65536KB chunk

unused devices: <none>

Saleen380:~ # date;dd if=/dev/zero of=/dev/pmem4 bs=1024 count=1024

Tue Nov 14 10:26:49 CST 2017

1024+0 records in

1024+0 records out

1048576 bytes (1.0 MB, 1.0 MiB) copied, 0.0015482 s, 677 MB/s

Saleen380:~ # date;mdadm --detail /dev/md0

Tue Nov 14 10:27:00 CST 2017

/dev/md0:

Version : 1.2

Creation Time : Fri Oct 20 06:30:49 2017

Raid Level : raid1

Array Size : 106889216 (101.94 GiB 109.45 GB)

Used Dev Size : 106889216 (101.94 GiB 109.45 GB)

Raid Devices : 2

Total Devices : 1

Persistence : Superblock is persistent

Intent Bitmap : Internal

Update Time : Tue Nov 14 10:23:19 2017

State : clean, degraded

Active Devices : 1

Working Devices : 1

Failed Devices : 0

Spare Devices : 0

Consistency Policy : bitmap

Name : Saleen380:0  (local to host Saleen380)

UUID : bf7e61b6:5a437ef7:88d06324:0aaac331

Events : 104

Number   Major   Minor   RaidDevice State

0     259        0        0      active sync   /dev/pmem2

-       0        0        1      removed

Saleen380:~ # date;mdadm --manage /dev/md0 --add /dev/pmem4

Tue Nov 14 10:27:07 CST 2017

mdadm: added /dev/pmem4

Saleen380:~ # date;mdadm --detail /dev/md0

Tue Nov 14 10:27:13 CST 2017

/dev/md0:

Version : 1.2

Creation Time : Fri Oct 20 06:30:49 2017

Raid Level : raid1

Array Size : 106889216 (101.94 GiB 109.45 GB)

Used Dev Size : 106889216 (101.94 GiB 109.45 GB)

Raid Devices : 2

Total Devices : 2

Persistence : Superblock is persistent

Intent Bitmap : Internal

Update Time : Tue Nov 14 10:27:12 2017

State : clean, degraded, recovering

Active Devices : 1

Working Devices : 2

Failed Devices : 0

Spare Devices : 1

Consistency Policy : bitmap

Rebuild Status : 1% complete

Name : Saleen380:0  (local to host Saleen380)

UUID : bf7e61b6:5a437ef7:88d06324:0aaac331

Events : 107

Number   Major   Minor   RaidDevice State

0     259        0        0      active sync   /dev/pmem2

2     259        3        1      spare rebuilding   /dev/pmem4

Saleen380:~ # date

Tue Nov 14 10:27:32 CST 2017

Saleen380:~ # sysctl -w dev.raid.speed_limit_min=100000

dev.raid.speed_limit_min = 100000

Saleen380:~ # sysctl -w dev.raid.speed_limit_max=500000

dev.raid.speed_limit_max = 500000

Saleen380:~ #

Saleen380:~ # date;mdadm --detail /dev/md0

Tue Nov 14 10:30:51 CST 2017

/dev/md0:

Version : 1.2

Creation Time : Fri Oct 20 06:30:49 2017

Raid Level : raid1

Array Size : 106889216 (101.94 GiB 109.45 GB)

Used Dev Size : 106889216 (101.94 GiB 109.45 GB)

Raid Devices : 2

Total Devices : 2

Persistence : Superblock is persistent

Intent Bitmap : Internal

Update Time : Tue Nov 14 10:30:42 2017

State : clean

Active Devices : 2

Working Devices : 2

Failed Devices : 0

Spare Devices : 0

Consistency Policy : bitmap

Name : Saleen380:0  (local to host Saleen380)

UUID : bf7e61b6:5a437ef7:88d06324:0aaac331

Events : 150

Number   Major   Minor   RaidDevice State

0     259        0        0      active sync   /dev/pmem2

2     259        3        1      active sync   /dev/pmem4

Saleen380:~ #

- Error injection script does not explain the mapping between device namespace and its NFIT mapping name. This is done using the region mapping. In the following output, first entry under MEMORY REGION maps to first entry under NAMESPACE. So, “namespace4” maps to “region4” and it in turn maps to “nmem3” used in the error injection script.

Saleen380:~/pmemeinj # date;./lsnvdimm  -r -n

Tue Nov 14 11:33:35 CST 2017

-- MEMORY REGIONS --

NAME    SIZE         NUMANODE MAPPINGS MAPPING0

region0 274877906944 0        1        nmem4,0,274877906944

region1 302795194368 0        1        nmem0,0,302795194368

region2 109521666048 0        1        nmem1,0,109521666048

region3 302795194368 1        1        nmem2,0,302795194368

region4 109521666048 1        1        nmem3,0,109521666048

-- NAMESPACES --

NAME              RESOURCE      SIZE         NUMANODE

namespace0.000000 0xa040000000  274877906944 0

namespace1.000000 0x59c0000000  302795194368 0

namespace2.000000 0x4040000000  109521666048 0

namespace3.000000 0x139c0000000 302795194368 1

namespace4.000000 0x12040000000 109521666048 1

Saleen380:~/pmemeinj #

- Startup SQL server and resume the StepMaster run.