Durability Test Transcript: Power Loss

This test demonstrates the ability of the database deployment to recover from an abrupt power loss. This solution uses persistent memory technology and involves redundant battery backed power modules to ensure persistence of the data. We depend on ILO event logs to narrate the various events of this power loss test.

Overview of powerless sequence from SUT standpoint:

- Power cables from the server are pulled-out to simulate abrupt power loss to the SUT.

- Power to the DIMMs is maintained using the battery backup.

- Server firmware ensures backup of the persistent memory devices’ contents.

- Server shuts down after the backup.

- Power cables are restored powering up iLO (management console)

- Server is powered up through the iLO.

- Server firmware restores the data on persistent memory devices after required validations and proceeds with normal startup.

Notes:

- With current test we have all iLO logs with time stamps from GMT time zone. (+6:00:00 from CT).

- iLO consolidates some of the consecutive log entries into one if these logs entries are similar/identical. The single log entry would have the ID of first log entry and time stamp of the latest log entry.

Following is the iLO event log dump for the entire test duration: (EventLog.csv in the log dump)

| ID | Severity | Description | Last Update | Count | Category |
| --- | --- | --- | --- | --- | --- |
| 9817 | Informational | Remote console session stopped by: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 14:01 | 1 | Administration |
| 9816 | Informational | Browser logout: [NONE] - 127.0.0.1(localhost). | 11/16/2017 13:55 | 1 | Security, Administration |
| 9815 | Informational | Browser logout: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:55 | 2 | Security, Administration |
| 9814 | Informational | Browser login: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:55 | 2 | Security, Administration |
| 9813 | Informational | Browser logout: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:51 | 2 | Security, Administration |
| 9812 | Informational | Browser login: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:51 | 2 | Security, Administration |
| 9811 | Informational | Browser logout: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:50 | 13 | Security, Administration |
| 9810 | Informational | Browser login: System Administrator - 127.0.0.1(localhost). | 11/16/2017 13:50 | 13 | Security, Administration |
| 9809 | Informational | Browser login: [NONE] - 127.0.0.1(localhost). | 11/16/2017 13:50 | 1 | Security, Administration |
| 9808 | Caution | Server reset. | 11/16/2017 13:49 | 1 | Maintenance, Administration |
| 9807 | Caution | Server reset. | 11/16/2017 13:49 | 1 | Maintenance, Administration |
| 9806 | Informational | Power-On signal sent to host server by: [NONE]. | 11/16/2017 13:49 | 1 | Maintenance, Administration, Power |
| 9805 | Informational | Remote console started by: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 13:49 | 1 | Administration |
| 9804 | Informational | APO: Restored power state to standby. | 11/16/2017 13:47 | 1 | Administration, Configuration |
| 9803 | Informational | Browser login: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 13:47 | 1 | Security, Administration |
| 9802 | Informational | iLO clock has been synchronized with 10.1.1.77. | 11/16/2017 13:52 | 1 | Configuration |
| 9801 | Informational | On-board clock set; was (dd/mm/yyyy) 1/1/1970 00:00:00. | 11/16/2017 13:47 | 1 | Administration, Configuration |
| 9800 | Informational | iLO network link up at 1000 Mbps. | [NOT SET] | 1 | Other |
| 9799 | Caution | Security jumper override detected. Security disabled! | [NOT SET] | 1 | Security |
| 9798 | Caution | Power restored to iLO. | [NOT SET] | 1 | Administration |
| 9797 | Caution | Server reset. | 11/16/2017 13:45 | 1 | Maintenance, Administration |
| 9796 | Informational | Remote console started by: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 13:43 | 1 | Administration |
| 9795 | Informational | Embedded Flash: Restarted. | 11/16/2017 13:43 | 1 | Administration |
| 9794 | Informational | Server power restored. | 11/16/2017 13:49 | 3 | Maintenance, Administration |
| 9793 | Caution | Server reset. | 11/16/2017 13:43 | 1 | Maintenance, Administration |
| 9792 | Informational | Event log cleared by: [NONE]. | 11/16/2017 13:41 | 1 | Maintenance, Administration |

Sequence of events:

- Invoke require workload from StepMaster client and wait for each of the 7 query streams to complete 100 iterations.

- Clear all the iLO event logs:

| 9792 | Informational | Event log cleared by: [NONE]. | 11/16/2017 13:41 | 1 | Maintenance, Administration |
| --- | --- | --- | --- | --- | --- |

- Pull the power cables from the server. iLO registers the “reset” event.

| 9793 | Caution | Server reset. | 11/16/2017 13:43 | 1 | Maintenance, Administration |
| --- | --- | --- | --- | --- | --- |

- Server firmware quickly triggers backup of persistent memory devices on battery power. Note that 3 log entries collapsed into one.

| 9795 | Informational | Embedded Flash: Restarted. | 11/16/2017 13:43 | 1 | Administration |
| --- | --- | --- | --- | --- | --- |
| 9794 | Informational | Server power restored. | 11/16/2017 13:49 | 3 | Maintenance, Administration |

- After the backup, server shutdown including iLO.

- Power cables are restored back to the server and iLO powers on automatically.

| 9802 | Informational | iLO clock has been synchronized with 10.1.1.77. | 11/16/2017 13:52 | 1 | Configuration |
| --- | --- | --- | --- | --- | --- |
| 9801 | Informational | On-board clock set; was (dd/mm/yyyy) 1/1/1970 00:00:00. | 11/16/2017 13:47 | 1 | Administration, Configuration |
| 9800 | Informational | iLO network link up at 1000 Mbps. | [NOT SET] | 1 | Other |
| 9799 | Caution | Security jumper override detected. Security disabled! | [NOT SET] | 1 | Security |
| 9798 | Caution | Power restored to iLO. | [NOT SET] | 1 | Administration |

- We log-in to iLO and launch “Remote Console” and power-on the server:

| 9806 | Informational | Power-On signal sent to host server by: [NONE]. | 11/16/2017 13:49 | 1 | Maintenance, Administration, Power |
| --- | --- | --- | --- | --- | --- |
| 9805 | Informational | Remote console started by: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 13:49 | 1 | Administration |
| 9804 | Informational | APO: Restored power state to standby. | 11/16/2017 13:47 | 1 | Administration, Configuration |
| 9803 | Informational | Browser login: [NONE] - 10.112.133.41(DNS name not found). | 11/16/2017 13:47 | 1 | Security, Administration |

- Server startups and restores all data on persistent memory devices:

| 9808 | Caution | Server reset. | 11/16/2017 13:49 | 1 | Maintenance, Administration |
| --- | --- | --- | --- | --- | --- |
| 9807 | Caution | Server reset. | 11/16/2017 13:49 | 1 | Maintenance, Administration |

- Server then completes its normal startup along with SQL server. All transactions are rolled forward during recovery:

- Workload from StepMaster is restarted and allowed to complete all checks/validations.