Hardware Configuration

8.3.1.4 A description of the steps taken to configure all the hardware must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.1). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the hardware environment

Driver

The driver system is not part of the System Under Test (SUT) and priced configuration. This system is connected to SUT using onboard LAN with 1Gb/s Ethernet. Another ethernet port is connected to 1GB/s Management network in the lab. This system has a 480GB SAS SSD configured with RAID 0 to install OS and a 960GB Samsung PM863a SATA SSD to store flat file. It has a AMD EPYC 7601 2.2GHz processor

System Under Test

The SUT is a Dell PowerEdge R7415 server with one AMD EPYC 7551P 2.0 GHz 32 core processor. 1x 480GB TOSHIBA SAS SSD configured with RAID 0 for OS. This sever has an integrated RAID controller PERC H330 Mini. This server has 2x internal Broadcom Gigabit Ethernet BCM5720, one of which is connected 1GB/s Management network in the lab and other is connected to Driver over 1GB/s Ethernet

Storage

16x Samsung PM863a SATA SSDs are used for data. The enclosures are connected to PERC H330 Mini Integrated RAID Controller and configured with RAID 10 to store data from both Tier A and Tier B virtual machines.

More information about installation and changes to the software, settings can be found in the SupportingFiles directory under Introduction – Hardware -Driver/SUT

Software Configuration

A description of the steps taken to configure all software must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.2). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the software environment.

The default installation of the operating system was executed on both driver and SUT server. More information about installation and changes to the software, settings can be found in the SupportingFiles directory under Introduction – Software – Driver/SUT