Hardware

- Hardware- For the Driver we used Dell R7415 Sever with following components:

  - Processor- 1 AMD EPYC 7601 32-Core Processor 2.20GHz

  - Memory- 8 x 16GB

  - Storage- 1x 480GB SSD(OS), 1x 960GB SSD (flat file)

  - Networking- 1x Broadcom Gigabit Ethernet BCM5720

- System Settings:

  - Access iDRAC web interface for the server and using console enter System setup in BIOS settings

  - Access the RAID controller and enter Configuration management to create virtual disks

  - Create a RAID 0 virtual disk with 480GB SSD and another RAID 0 virtual disk with 960GB SSD.

  - From iDRAC console connect virtual media and map CD/DVD and mount esxi 6.5 iso

  - Select virtual CD/DVD at next boot and continue with Esxi installation process

  - Select RAID 0 480GB SSD as bootable device and install Esxi with default settings

- Network

  - Connect server’s management ethernet port to the lab network switch so that another system on the lab network can now manage the server