Hardware

- Hardware - For the Driver we used HPE DL385 Sever with following components:

  - Processor- 2x AMD EPYC 7601 32-Core Processor 2.20GHz

  - Memory- 4 x 32GB

  - Storage- 1x 240GB SSD(OS), 2x 960GB SSD (flat files)

  - Networking- 1x HPE Ethernet 1Gb 4-port 331i Adapter, 1x HPE 10/25Gb 2p 640FLR-SFP28 Adptr

- System Settings:

  - Access iLO web interface for the server and using console enter System setup in BIOS settings

  - Access the RAID controller using HPE Smart Storage Administrator to create virtual disks

  - Create a RAID 0 virtual disk with 240GB SSD and another RAID 0 virtual disk with 2x 960GB SSD.

  - From iLO console connect virtual media and map CD/DVD and mount esxi 6.7 U2 iso

  - Select CD/DVD at next boot through Administration -> Boot Order and install esxi

  - Select RAID 0 240GB SSD as bootable device and install esxi with default settings

- Network

  - Connect management ethernet port to the lab network switch, that way another system can now access the client