Hardware - SUT

- Hardware - For the SUT we used HPE DL385 Gen10 Sever with following components:

  - Processor- 2x AMD EPYC 7702 2.0 GHz

  - Memory- 16 x 64GB

  - Storage- 1x 40GB SSD(OS), 16x 1.92TB SSD (Data), 8x 3.84TB SSD (Data)

  - Networking- 1x HPE Ethernet 1Gb 4-port 331i Adapter, 1x HPE 10/25Gb 2p 640FLR-SFP28 Adptr

- System Settings:

  - Access iLO web interface for the server and using console enter System setup in BIOS settings

  - Access the RAID controller using HPE Smart Storage Administrator to create virtual disks

  - Create a RAID0 virtual disk with 240GB SSD and one RAID10 virtual disk with 16x 1.92TB SSD and another RAID10 virtual disk with 8x 3.84TB SSD.

  - From iLO console connect virtual media and map CD/DVD and mount esxi 6.5 U3 iso

  - Select CD/DVD at next boot through Administration -> Boot Order and install esxi

  - Select RAID0 240GB SSD as bootable device and install esxi with default settings

- Network

  - Connect management ethernet port to the lab network switch, that way client can now access the SUT