Hardware Configuration

8.3.1.4 A description of the steps taken to configure all the hardware must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.1). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the hardware environment

Driver

The driver system is not part of the System Under Test (SUT) and priced configuration. This system is connected to SUT using Embedded LOM 10Gb/s Ethernet. Another ethernet port is connected to 1GB/s Management network in the lab. This system has a 240GB SATA M.2 SSD configured with RAID0 to install OS and 2x 960GB Samsung PM863a SATA SSD to store flat files. It has 2x AMD EPYC 7601 2.2GHz processor

System Under Test

The SUT is an HPE ProLiant DL385 Gen10 server with 2x AMD EPYC 7702 2.0 GHz 64 core processor. 1x 240GB Micron SATA M.2 SSD configured with RAID0 for OS. This sever has one HPE Smart Array P408i-p SR Gen10 RAID controller, HPE Smart Array P816i-a SR Gen10 RAID controller. This server has 1x embedded HPE Ethernet 1Gb 4-port 331i Adapter & 1x HPE Eth 10/25Gb 2p 640FLR-SFP28 Adapter, one of which is connected 1GB/s Management network in the lab and other is connected to Driver over 10GB/s Ethernet

Storage

16x Samsung 2.5" PM883 1.92TB SATA SSDs, 6x Samsung 2.5" PM883 3.84TB SATA SSD, 2x Micron 5200 ECO 2.5" 3.84TB SATA SSD are used for data. 16x 1.92TB drives are connected to HPE Smart Array P816i-a SR Gen10 RAID controller and 8x 3.84TB drives are connected to HPE Smart Array P816i-a SR Gen10 RAID controller and configured with RAID 10 to store data from both Tier A and Tier B virtual machines.

More information about installation and changes to the software, settings can be found in the SupportingFiles directory under Introduction – Hardware -Driver/SUT

Software Configuration

A description of the steps taken to configure all software must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.2). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the software environment.

The default installation of the operating system was executed on both driver and SUT server. More information about installation and changes to the software, settings can be found in the SupportingFiles directory under Introduction – Software – Driver/SUT