Hardware Configuration

8.3.1.4 A description of the steps taken to configure all the hardware must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.1). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the hardware environment

Driver

The driver system is not part of the System Under Test (SUT) and priced configuration. This system is connected to SUT using Integrated 200Gb/s Ethernet Adapter. Another Ethernet port is connected to 1GB/s Management network in the lab. This system has 2 x 480GB PM883 SSD configured with RAID 1 to install OS and 4 x 6.4TB NVMe to store flat files and results. It has 2 x AMD EPYC 9754 Processor.

System Under Test

The SUT is a InspurCloud ICP Edge ICP5220A4 Sever with 2x AMD EPYC 9754 2.25 GHz 128 core processor. 2x 960GB PM983 M.2 SSD configured with RAID 1 for OS. This sever has 3 x MegaRAID LSI-9560-16i(8G) RAID controllers. This server has 1x embedded 1Gb Intel Gigabit Ethernet – NIC adapter & 1x Mellanox 200Gb dual-port QSFP Adapter, the former is connected 1GB/s Management network in the lab and other is connected to Driver over 200GB/s Ethernet.

Storage

12 x  D7-P5620 NVMe U.2 12.8TB are used for data. 4 x 12.8TB drives are connected to one of the MegaRAID LSI-9560-16i(8G) RAID controller, 4 x drives are connected to the second RAID controller and all the left 4 x drives are connected to the third controller and all the controllers are configured with RAID 10 to store data from both Tier A and Tier B virtual machines.

More information about installation and changes to the software, settings can be found in the Supporting Files directory under Introduction – Hardware -Driver/SUT

Software Configuration

A description of the steps taken to configure all software must be reported in the Report. All configuration scripts or step-by-step GUI instructions are reported in the Supporting Files (see Clause 8.4.1.2). The description, scripts and GUI instructions must be enough such that a reader knowledgeable of computer systems and the TPCx-V specification could recreate the software environment.

The default installation of the operating system (InLinux 23.12 LTS) was executed on both driver and SUT server. More information about installation and changes to the software, settings can be found in the Supporting Files directory under Introduction – Software – Driver/SUT